S Version Upstream & Downstream Communication Protocol (V1.3) # （For Observation & Aiming Thermal Imaging Module）

## 1. Version management Revision No.; Date; Revision; Sig.. V1.0; 202501; Initial Release. V1.1; 202502; Corrections to image mirroring commands. V1.2; 202503; Corrections to manual focus. V1.3; 202512; Image Algorithm Optimization Defect Cursor Toggle.

Table of Contents 1 Product Overview 3 1.1 Protocol Description 3 2 General Downlink Protocol 3 2.1 Image 4 2.1.1 Image Mode 4 2.1.2 Video Mode Control 5 2.2 Diagnostics 7 2.3 Lens/Calibration 8 2.3.1 Lens 8 2.3.2 Non-uniformity Correction 9 3 General Uplink Protocol (Query Protocol) 10 3.1 Status Page Query 10 3.1.1 Send Query Protocol 10 3.1.2 Response Protocol Format 10 3.2 Digital Video Page Query 11 3.2.1 Send Query Protocol 11 3.2.2 Response Protocol Format 12 3.3 Settings Page Query 14 3.3.1 Send Query Protocol 14 3.3.2 Response Protocol Format 14 3.4 Focus Page 15 3.4.1 Send Query Protocol 15 3.4.2 Response Protocol Format 15

# 1 Product Overview This protocol is applicable to imaging-type thermal imaging products. ## 1.1 Protocol Description - After successful command transmission, the device returns success frame: 55 AA 01 00 01 F0 If reception error occurs, return retransmission request frame: 55 AA 01 01 00 F0. - XOR bitwise checksum: Checksum calculation starts from the byte after 55 AA. Example: Shutter compensation command55 AA 07 02 01 08 00 00 00 01 0D F0 Checksum = 07 XOR 02 XOR 01 XOR 08 XOR 00 XOR 00 XOR 00 XOR 01 = 0D. 2 General Downstream Protocol Note: 1. All modified parameters must be saved by sending Save Settings command to take effect permanently. 2. Shutter compensation and scene compensation are executed automatically by default.For manual scene compensation operation method, please consult business or technical support.Pseudo-color switching is only available for cool tone palette mode. General; Features; Commands; Remark. Compensation; Shutter Compensation; 55 AA 07 02 01 08 00 00 00 01 0D F0. Scene Compensation; 55 AA 07 02 01 07 00 00 00 01 02 F0. Image; Image Freeze On; 55 AA 07 01 00 02 00 00 00 01 05 F0. Image Freeze Off; 55 AA 07 01 00 02 00 00 00 00 04 F0. False-color; White Heat; 55 AA 07 02 00 04 00 00 00 00 01 F0. Black Heat; 55 AA 07 02 00 04 00 00 00 09 08 F0. Red Outline; 55 AA 07 02 00 04 00 00 00 08 09 F0.

Lava; 55 AA 07 02 00 04 00 00 00 01 00 F0. Iron Red; 55 AA 07 02 00 04 00 00 00 02 03 F0. Hot Iron; 55 AA 07 02 00 04 00 00 00 03 02 F0. Medical; 55 AA 07 02 00 04 00 00 00 04 05 F0. Arctic; 55 AA 07 02 00 04 00 00 00 05 04 F0. Rainbow 1; 55 AA 07 02 00 04 00 00 00 06 07 F0. Rainbow 2; 55 AA 07 02 00 04 00 00 00 07 06 F0. ## 2.1 Image 2.1.1 Image Mode 2.1.1.1 Image Tone Image Tone; Features; Commands; Remark. Warm colors; 55 AA 07 02 02 19 00 00 00 00 1E F0. Cool colors; 55 AA 07 02 02 19 00 00 00 01 1F F0. Green heat; 55 AA 07 02 02 19 00 00 00 02 1C F0. 2.1.1.2 Image Palette Mode Image Mode; Function; Commands; Remark. Soft; 55 AA 07 02 02 06 00 00 00 00 01 F0. Standard; 55 AA 07 02 02 06 00 00 00 01 00 F0. Highlight; 55 AA 07 02 02 06 00 00 00 03 02 F0.

Noise Reduction (1–3); 55 AA 07 02 02 1C 00 00 00 xx XOR F0. Detail Enhancement (1–4); 55 AA 07 02 02 1D 00 00 00 xx XOR F0. Brightness (1–5); 55 AA 07 02 02 1E 00 00 00 xx XOR F0. Contrast (1–5); 55 AA 07 02 02 1F 00 00 00 xx XOR F0. 2.1.1.3 Image Mirror & Electronic Zoom Image Mirroring; No; 55 AA 07 02 00 05 00 00 00 00 00 F0; Remark. Mirror in the X direction; 55 AA 07 02 00 05 00 00 00 01 01 F0. Mirror in the Y direction; 55 AA 07 02 00 05 00 00 00 02 02 F0. Mirror in the XY direction; 55 AA 07 02 00 05 00 00 00 03 03 F0. Image Zoom; 8–64x (actually corresponds to 1–8); 55 AA 07 02 00 06 00 00 00 xx XOR F0. ### 2.1.2 视频模式控制 2.1.2.1 Analog Video Analog Video; Functions; Commands; Remark. Analog Video; Video Off; 55 AA 07 02 00 01 00 00 00 00 04 F0. Video On; 55 AA 07 02 00 01 00 00 00 01 05 F0. PAL System; 55 AA 07 02 00 02 00 00 00 02 05 F0. Analog video formats; NTSC system; 55 AA 07 02 00 02 00 00 00 03 04 F0.

Note：Before using the analog video feature, please check with your sales or technical support team to confirm whether this product supports this feature. PAL has a resolution of 720×576, while NTSC has a resolution of 720×480. 2.1.2.2 Digital Video l Frame rate Note: Before changing the digital video frame rate, please consult your account manager or technical support to confirm whether this product supports this feature. Frame rate; Function; Commands; Remark. 50HZ; 55 AA 07 02 01 05 00 00 00 03 02 F0; 1k resolution not supported. 30HZ; 55 AA 07 02 01 05 00 00 00 00 01 F0. 25HZ; 55 AA 07 02 01 05 00 00 00 01 00 F0. l Digital Port Type Note: Before changing the content of a digital port, please consult your business or technical support team to confirm whether the product supports this feature. Digital Port Type; Functions; Commands; Remark. Off; 55 AA 07 02 01 02 00 00 00 00 06 F0. USB2.0; 55 AA 07 02 01 02 00 00 00 01 07 F0. CMOS; 55 AA 07 02 01 02 00 00 00 02 04 F0. BT656; 55 AA 07 02 01 02 00 00 00 04 02 F0. BT1120; 55 AA 07 02 01 02 00 00 00 03 05 F0; Only 1k Resolution support. LVDS; 55 AA 07 02 01 02 00 00 00 07 01 F0. USB2.0+UART; 55 AA 07 02 01 02 00 00 00 05 03 F0. LCD; 55 AA 07 02 01 02 00 00 00 06 00 F0. LCD+DVP; 55 AA 07 02 01 02 00 00 00 08 0E F0. UVC + CDC; 55 AA 07 02 01 02 00 00 00 09 0F F0. DSI; 55 AA 07 02 01 02 00 00 00 0B 0D F0; MIPI-DSI.

l CMOS Content Type; Function; Commands; Remark. Mode; YUV422; 55 AA 07 02 01 03 00 00 00 00 07 F0. YUV422+ Parameter line; 55 AA 07 02 01 03 00 00 00 01 06 F0. Y16; 55 AA 07 02 01 03 00 00 00 02 05 F0. Y16+Parameter line; 55 AA 07 02 01 03 00 00 00 03 04 F0. 2.2 USB Switch This function enables / disables USB3.0 (HID + UVC) function.Setting takes effect after saving configuration and restarting the thermal core. Type; Function; Commands; Remark. Mode; USB Off; 55 AA 07 02 01 06 00 00 00 00 02 f0. USB On; 55 AA 07 02 01 06 00 00 00 01 03 f0. 2.3 Diagnosis Type; Features; Commands; Remark. Parameters; Save Settings; 55 AA 07 01 00 04 00 00 00 01 03 F0. Parameters; Restore Factory Settings; 55 AA 07 01 00 05 00 00 00 01 02 F0.

## 2.4 Lens / Calibration ### 2.4.1 Lens l Optical Zoom Note: Only zoom lenses support this feature. After using Zoom In or Zoom Out, you must use the Zoom Stop command. Zoom; Function; Commands; Remark. Zoom In; 55 AA 07 03 00 07 00 00 00 01 02 F0. Zoom Out; 55 AA 07 03 00 07 00 00 00 02 01 F0. Zoom Stop; 55 AA 07 03 00 07 00 00 00 00 03 F0. l Optical focusing Note: Only lenses with electric focus support this feature. To use Telephoto+ or Telephoto-, you must use the focus stop command. Focusing; Function; Commands; Remark. Telephoto+; 55 AA 07 03 00 06 00 00 00 01 03 F0. Telephoto-; 55 AA 07 03 00 06 00 00 00 02 00 F0. Focus Hold; 55 AA 07 03 00 06 00 00 00 00 02 F0. Manual Focus Speed; 55 AA 07 03 00 02 00 00 00 XX XOR F0. 1–10. Autofocus; 55 AA 07 03 00 06 00 00 00 03 01 F0.

### 2.4.2 Non-uniformity correction l Shutter Function; Commands; Remark. Shutter; Close; 55 AA 07 A0 02 08 00 00 00 00 AD F0. Expand; 55 AA 07 A0 02 08 00 00 00 01 AC F0. Adaptive compensation; Open; 55 AA 07 01 00 07 00 00 00 01 00 F0. Close; 55 AA 07 01 00 07 00 00 00 00 01 F0. Time Setting （0~100min(0 Indicates that the shutter will not be released from this point on)）; 55 AA 07 01 00 01 00 00 00 xx XOR F0. Note: Do not turn off the adaptive compensation switch, as this will prevent the system from automatically adjusting the image, which may affect image quality. l Dead Pixel Correction Function; Commands; Remark. Dead pixel cursor; On; 55 AA 07 03 01 01 00 00 00 01 05 F0. Off; 55 AA 07 03 01 01 00 00 00 00 04 F0. dead pixel; Dead pixel X-coordinate; 55 AA 07 03 01 02 00 00 xx xx XOR F0. （0~width-1）. Defect Y-coordinate (0–height–1); 55 AA 07 03 01 03 00 00 xx xx XOR F0. Add defect; 55 AA 07 03 01 04 00 00 00 01 00 F0. Save defects; 55 AA 07 03 01 05 00 00 00 01 01 F0.

（0~width-1）. Enter Y coordinate (0–height–1); 55 AA 07 03 01 03 00 00 xx xx XOR F0. Add bad pixel; 55 AA 07 03 01 04 00 00 00 01 00 F0. Save bad pixel; 55 AA 07 03 01 05 00 00 00 01 01 F0. # 3 General Upstream Protocol (Query Protocol) 3.1 Status Page Query ### 3.1.1 Query Command Transmission Function; Commands; Remark. Status Page Query; 55 AA 07 00 00 80 00 00 00 00 87 F0. ### 3.1.2 Response Protocol Format Byte; commands; Parameter Description; Remark. Byte0; 0x55; Frame Header Byte 1. Byte1; 0xAA; Frame Header Byte 2. Byte2; 0x13; Length 19. Byte3; 0x00; Status Page (Function Category). Byte4; 0x00; Page 1 (Page Number). Byte5; xx; Product ID and Product Type.

byte6; xx; TBD. Byte7; Year; Firmware version number. Byte8; Month. Byte9; Date. Byte10; xx; Highest 8 digits of coke temperature. Byte11; xx; Lowest 8 digits of coke temperature. Byte12~byte13; xx; TBD. Byte14; xx; Machine ID [31:24]. Byte15; xx; Machine ID [23:16]. Byte16; xx; Machine ID [15:8]. Byte17; xx; Machine ID [7:0]. Byte18~ Byte21; xx; TBD. Byte22; xx; Checksum. Byte23; 0XF0; Frame End. Eg： 55 aa 13 00 00 2e 00 17 0a 11 0e 30 02 01 8f 3c da 97 01 04 03 00 f4 f0 Temperature: 0e 30 represents 3632/100 = 36.32°C Firmware version: 17 0a 11 represents 231017 Device ID: 8f 3c da 97 represents 2403130007 ## 3.2 Digital Video Page Query Features; Commands; Remark. Digital Page Search; 55 AA 07 02 01 80 00 00 00 00 84 F0. ### 3.2.1 Query Command Transmission

### 3.2.2 Response Protocol Format Byte; Commands; Parameter description; Remark. Byte0; 0x55; Frame Header Byte 1. Byte1; 0xAA; Frame Header Byte 2. Byte2; 0x13; Length 19. Byte3; 0x02; Status Page (Function Classification). Byte4; 0x01; Digital Video Page. Byte5; 0x00; Reserved. Byte6; 0x00; Digital Port Parallel Port Off. 0x01; USB2.0. 0x02; CMOS. 0x03; BT1120. 0x04; BT656. 0x05; USB2.0+UART. 0x06; LCD. 0x07; LVDS. 0x08; LCD + DVP. 0x09; UVC + CDC.

Byte7; 0x00; YUV422. 0x01; YUV422+Parameter Line. 0x02; Y16. 0x03; Y16+Parameter Line. 0x04; Reserved. 0x05; Reserved. 0x08; Reserved. 0x09; Reserved. 0x0A; Reserved. 0x0B; Reserved. Byte8; 0x00; Reserved. 0x01; Reserved. 0x02; Reserved. Byte9; 0x00; 30HZ. 0x01; 25HZ. 0x02; Reserved. 0x03; 50HZ. Byte10; xx; TBD. Byte11; 0x00; Rising-edge alignment. 0x01; Falling-edge alignment. Byte12- Byte21; xx; TBD.

Byte22; xx; Checksum. Byte23; 0XF0; Frame End. Eg: 55 aa 13 02 01 00 01 05 01 00 00 01 00 00 00 00 00 00 00 00 00 00 14 f0 External Synchronization Mode：00 =Off Digital Port Type：01 = USB2.0 CMOS content：05=Y16+Parameter line+YUV422 CMOS Interface：01 = CMOS8(MSB) Frame rate：00 = 30hz Clock phase：01=falling edge 3.3 Setting Page Query ### 3.3.1 Query Command Transmission Function; Commands; Remark. Setting Page Query; 55 AA 07 01 00 80 00 00 00 00 86 F0. ### 3.3.2 Response Protocol Format Byte; Commands; Parameter Description; Remark. Byte0; 0x55; Frame Header Byte 1. Byte1; 0xAA; Frame Header Byte 2. Byte2; 0x13; Length 19. Byte3; 0x01; Configuration Page (Function Category). Byte4; 0x00; Page 1 (Page Number). Byte5; xx; Automatic Time Compensation. Byte6~byte7; xx; TBD. Byte8; 0x00; Temperature Rise Compensation Switch Off.

0x01; Temperature rise compensation switch on. Byte9~Byte21; xx; TBD. Byte22; xx; Checksum. Byte23; 0XF0; End of frame. 3.4 Focus Page Query ### 3.4.1 Query Command Transmission Features; Commands; Remark. Search the Settings Page; 55 AA 07 03 00 80 00 00 00 00 84 f0. ### 3.4.2 Response Protocol Format Byte; Commands; Parameter description; Remark. Byte0; 0x55; Frame Header Byte 1. Byte1; 0xAA; Frame Header Byte 2. Byte2; 0x13; Length 19. Byte3; 0x03; Status Page (Function Classification). Byte4; 0x00; Focus Page. Byte5; 0xxx; Lens Type. Byte6~ Byte21; xx; TBD. Byte22; xx; Checksum. Byte23; 0XF0; Frame Trailer.